SNA Series

Overview

The SNA series capacitors with double side metallized

carrier film with internal series connection and metalized

polypropylene film, polyester wrapping with epoxy

resin fill.

Application:

The SNA series is designed for snubber/pulse applications.

For high pulse and high frequency applications requiring

extremely reliable contacts e.g. IGBT-applications

Benefits

Self-healing

Very low dissipation factor

Very high ripple current

Internal series connection

Negative capacitance change versus temperature

Construction

Dielectric: Polypropylene (PP) film.

Capacitor electrodes: Double-sided metallized plastic film.

Protection: Polyester wrapping with epoxy resin fill.

Terminals: Tinned copper wire (lead-free).

Internal construction:

Structure of ordering code

SNA – 1200 - 0.47 – 44  D

1     2   3    4   5

1  Series code

2  Rated voltage: 1200 V

3  Rated capacitance: 0.47 μF

4  Length of capacitor

5  D for round axial, F for flat oval

SNA Series

Reference standards                         IEC 61071 , IEC 60068 , RoHS compliance.

Degree of protection                         IP00

Rated capacitance (CN)                       0.047μF ... 6.8μF

Rated voltage (UNDC)                         700V ... 3000V

Standard capacitance tolerance                 K: ±10% , J: ±5%

Dissipation factor tan δ (1kHz@20°C)             ≤ 5.0 • 10-4

Test voltage between terminals UTT               1.5 UNDC , 10s

Test voltage between terminals and case UTC        4000 VAC, 10s

Insulation Resistance                      30000s but need not exceed 30GW (typical value),

after 1 minute of electrification at 100Vdc (25 ± 5°C).

Self inductance                        ≤1nH/mm of capacitor and leads length used for connection

Operating temperature range (case)          –40 °C ... +105°C

Storage temperature Θstg                 –40 °C ... +105 °C

Climatic category                        40/105/56

Capacitance deviation                   in the operating Temperature range of -40 to +85 °C,±1.5%

max on capacitance value measured at +20 °C

Expected lifetime                       100 000 h at UNDC @ Θhs 85°C

Capacitance drop at end of life              -3% (typical)

Fit rate                               50 (100 000 h at UNDC @ at Θhs 85°C)

SNA  Designs

SNA Series

UN  3000V      Us  4500V      Urms  1000V

| Ordering code | Cn (μF) | DIMENSIONS (mm) |  |  |  |  |  | du/dt (V/μs) | Ipk (A) | Rs @100kHz (mΩ) | Imax 20kHz@70° (A) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Designs D |  | Designs F |  |  | d |  |  |  |  |
|  |  | D | L | W | T | L |  |  |  |  |  |
| SNA-3000-0.047-44D | 0.047 | 14.5 | 44 |  |  |  | 1.0 | 1780 | 84 | 23.8 | 6 |
| SNA-3000-0.047-44F | 0.047 |  |  | 17.5 | 9.5 | 44 | 1.0 | 1780 | 84 | 23.8 | 6 |
| SNA-3000-0.068-44D | 0.068 | 17 | 44 |  |  |  | 1.0 | 1780 | 121 | 16.5 | 8 |
| SNA-3000-0.068-44F | 0.068 |  |  | 20 | 12 | 44 | 1.0 | 1780 | 121 | 16.5 | 8 |
| SNA-3000-0.10-44D | 0.10 | 20 | 44 |  |  |  | 1.2 | 1780 | 178 | 11.6 | 11 |
| SNA-3000-0.10-44F | 0.10 |  |  | 24 | 14.5 | 44 | 1.2 | 1780 | 178 | 11.6 | 11 |
| SNA-3000-0.15-44D | 0.15 | 24.5 | 44 |  |  |  | 1.2 | 1780 | 267 | 7.8 | 12 |
| SNA-3000-0.15-44F | 0.15 |  |  | 28.5 | 19 | 44 | 1.2 | 1780 | 267 | 7.8 | 12 |
| SNA-3000-0.22-57D | 0.22 | 22.5 | 57 |  |  |  | 1.2 | 1160 | 255 | 10.6 | 12 |
| SNA-3000-0.22-57F | 0.22 |  |  | 26 | 16.5 | 57 | 1.2 | 1160 | 255 | 10.6 | 12 |
| SNA-3000-0.33-57D | 0.33 | 27 | 57 |  |  |  | 1.2 | 1160 | 383 | 7.3 | 12 |
| SNA-3000-0.33-57F | 0.33 |  |  | 31.5 | 20.5 | 57 | 1.2 | 1160 | 383 | 7.3 | 12 |
| SNA-3000-0.47-57D | 0.47 | 32 | 57 |  |  |  | 1.2 | 1160 | 545 | 5.2 | 12 |
| SNA-3000-0.47-57F | 0.47 |  |  | 36.5 | 25.5 | 57 | 1.2 | 1160 | 545 | 5.2 | 12 |
| SNA-3000-0.068-57D | 0.68 | 38.5 | 57 |  |  |  | 1.2 | 1160 | 789 | 3.7 | 12 |
| SNA-3000-0.068-57F | 0.68 |  |  | 43.5 | 31 | 57 | 1.2 | 1160 | 789 | 3.7 | 12 |

(1) Maximum values at 100kHz, +60°C for case operating T= +85°C

(2) Typical values at 100kHz.